| Wat Bang Udom (New) Site Name Wat Bang Udom (New) Latitude 8.20579 Longitude 100.25374 Elevation(m) 4.0 TX(MHz) NA Polarization Dual-Polarized Antenna Model A7WD09MAC-3NX Diversity Antenna Model Antenna Height(m) 50.0 Diversity Antenna Height(m) 0.0 Antenna Diameter(m) 0.9 Diversity Antenna Diameter(m) 0.0 | Frequency = 7575.00 Protection = 2+0 Distance(km) = 18.416 XPIC = Enable Radio Model = 7G_XMC3E_32Q_28M_112M Capacity(M) = 224.0 Thermal Fade Margin(dB) = 42.78 RX Signal(dBm) = -36.21 Annual Availability(%) = 99.999486 Site1 Annual Multipath Availability(%) = 99.999876 Site2 Annual Multipath Availability(%) = 99.999876 Annual Rain Availability(%) = 99.999733 | ที่ว่าการอำเภอหัวไทร Site Name ที่ว่าการอำเภอหัวไทร Latitude 8.04059 Longitude 100.27467 Elevation(m) 2.19 TX(MHz) NA Polarization Dual-Polarized Antenna Model A7WD09MAC-3NX Diversity Antenna Model Antenna Height(m) 50.0 Diversity Antenna Height(m) 0.0 Antenna Diameter(m) 0.9 Diversity Antenna Diameter(m) 0.0 |
| --- | --- | --- |

| Link Id | จุด A สถานีต้นทาง |  |
| --- | --- | --- |
| Link Name | จุด A สถานีต้นทาง |  |
| Site Id | (New) | จุด A สถานีต้นทาง |
| Site Name | (New) | จุด A สถานีต้นทาง |
| Elevation(m) Latitude Longitude Antenna azimuth(°) Vertical angle(°) | 4.0 8.20579 100.25374 172.8511 -0.01 | 2.19 8.04059 100.27467 352.8511 0.01 |
| Antenna S to D | NO |  |
| Antenna model Antenna height(m) Antenna gain(dBi) | A7WD09MAC-3NX 50.0 34.9 | A7WD09MAC-3NX 50.0 34.9 |
| Frequency(MHz) Polarization Path length (km) Free space loss(dB) Atmospheric absorption loss (dB) Field margin(dB) Diffraction loss(dB) | 7575.0 H+V 18.416 135.33 0.19 1.0 0.0 |  |
| AM Status | Enable |  |
| ATPC Status | Disable |  |
| CA Mode | Disable |  |
| Power Booster | NA |  |
| Radio model Radio file name Emission designator TX channels RX threshold criteria | 7G_XMC3E_32Q_28M_112M 7GXMC3E32Q28MX 28M0D7W NA 10-6 | 7G_XMC3E_32Q_28M_112M 7GXMC3E32Q28MX 28M0D7W NA 10-6 |
| Average annual temperature(°C) | 25.0 |  |
| LogK Worst month multipath outage (sec) | -5.35041 11.07 | -5.35041 11.07 |
| 0.01% Rain rate(mm/hr) Rain region Link rain attenuation(dB) | 95.0 N 12.03 |  |

| Modulation mode | TX power (dBm) |  | RX threshold level (dBm) |  | EIRP (dBm) |  | Receive signal (dBm) |  | TR-DR signal (dBm) |  | Flat fade margin - multipath (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 512QAM 424.0Mbps 256QAM 380.0Mbps 128QAM 334.0Mbps 64QAM 282.0Mbps 32QAM 224.0Mbps | 28.5 28.5 30.5 30.5 30.5 | 28.5 28.5 30.5 30.5 30.5 | -60.3 -63.6 -66.6 -69.9 -79.0 | -60.3 -63.6 -66.6 -69.9 -79.0 | 63.4 63.4 65.4 65.4 65.4 | 63.4 63.4 65.4 65.4 65.4 | -38.21 -38.21 -36.21 -36.21 -36.21 | -38.21 -38.21 -36.21 -36.21 -36.21 | 0.0 0.0 0.0 0.0 0.0 | 0.0 0.0 0.0 0.0 0.0 | 20.11 22.66 25.43 27.3 33.95 | 20.11 22.66 25.43 27.3 33.95 |
| Modulation mode | Thermal fade margin (dB) |  | Worst month multipath (%) |  | Annual multipath (%) |  | Annual rain (%) |  | Total annual (%) |  | Time in mode (%) |  |
| 512QAM 424.0Mbps 256QAM 380.0Mbps 128QAM 334.0Mbps 64QAM 282.0Mbps 32QAM 224.0Mbps | 22.08 25.38 30.38 33.68 42.78 | 22.08 25.38 30.38 33.68 42.78 | 99.989788 99.994324 99.996998 99.998046 99.999573 | 99.989788 99.994324 99.996998 99.998046 99.999573 | 99.997042 99.998356 99.99913 99.999434 99.999876 | 99.997042 99.998356 99.99913 99.999434 99.999876 | 99.995417 99.997208 99.998546 99.999066 99.999733 | 99.995417 99.997208 99.998546 99.999066 99.999733 | 99.989502 99.993921 99.996807 99.997934 99.999486 |  | 99.989502 0.004419 0.002886 0.001127 0.001552 |  |
| Modulation mode | XPD threshold degradation - multipath(dB) |  |  |  | XPD threshold degradation - rain(dB) |  |  |  | SD Improvement Factor |  |  |  |
| 512QAM 424.0Mbps 256QAM 380.0Mbps 128QAM 334.0Mbps 64QAM 282.0Mbps 32QAM 224.0Mbps | 1.96 2.71 4.94 6.37 8.82 |  | 1.96 2.71 4.94 6.37 8.82 |  | 2.11 2.02 2.17 1.98 0.86 |  | 2.11 2.02 2.17 1.98 0.86 |  | 1.0 1.0 1.0 1.0 1.0 |  | 1.0 1.0 1.0 1.0 1.0 |  |